[ARTIST NAME]: OFFICIAL PERFORMANCE TECHNICAL RIDER

Contact: J. Lebang | www.jlebang.com| Spotify | Facebook

1. STAGE REQUIREMENTS

- Minimum Stage Dimensions: 36’ Wide x 22’ Deep (Required for full ensemble and horn section).

- Risers:

  - Drum Riser: 8’x8’ (12”–18” height).

  - Horn Section Riser: 8’x4’ (12” height) positioned Stage Right.

  - Guitar Riser: 8’x4’ (12” height) positioned Stage Left.

- Power: Minimum of six (6) separate 20-amp circuits with quad-boxes distributed across the downstage, upstage rhythm section, and horn risers.

2. FRONT OF HOUSE (FOH) & SOUND SYSTEM

- PA System: Professional 3-way or 4-way active line array system (L'Acoustic, Meyer, or Adamson preferred) capable of delivering 110dB of undistorted audio at the FOH position.

- Console: 48-channel digital console (

Midas M32

,

Yamaha CL5

, or

Allen & Heath dLive

preferred).

- Effects: Required high-quality digital reverb, delay, and parallel compression for the main vocals.

3. INPUT LIST (SUMMARY)



| Channel | Instrument | Preferred Mic/DI |
| --- | --- | --- |
| 1-8 | Drums (Kick, Snare, Toms, OH) | Shure Beta 52 / SM57 / SM81 |
| 9-10 | Bass Guitar (DI & Mic) | Radial J48 DI / Sennheiser MD421 |
| 11-14 | Electric Guitars (L/R) | Shure SM57 or Sennheiser e906 |
| 15-18 | Detroit Wind Section (Sax, Horn, Trpt, Clarinet) | Clip-on Condensers (preferred) or SM57 |
| 19-20 | Keyboards (Stereo DI) | Radial ProD2 |
| 21-26 | Vocals (Lead & Backing) | Shure Beta 58 Wireless |

4. MONITORING

- IEMs: Four (4) channels of wireless in-ear monitoring for lead vocalists and rhythm core.

- Wedges: Minimum of six (6) high-power floor monitors (12” or 15”) on separate mixes:

  - Mix 1 (Lead): Vocals, Guitars, slight Snare.

  - Mix 2 (Horns): Full Horn mix, Piano, Kick.

  - Mix 3 (Rhythm): Kick, Bass, Rhythm Guitars.

5. LOAD-IN & SOUNDCHECK

- Staffing: Venue to provide minimum three (3) experienced stagehands for load-in/out.

- Soundcheck: A minimum of two (2) hours for a full-system soundcheck following load-in. Venue must be clear of audience during this time.

6. HOSPITALITY

- Dressing Room: One (1) private, climate-controlled room with seating for 10.

- Refreshments: Bottled water (room temp), coffee, assorted protein/fruit plate, and a selection of local craft beers (non-IPA preferred).